KEGERATOR GUIDE

1. Keep fridge upright, allow settling time if tipped on it’s side before turning it on.

2. Allow kegs to settle if shaken excessively.

3. Attach wheels to fridge if desired.

4. Attach tower to top using small screws (may be inside fridge). Ensure gasket is in place. Assemble taps and screw on.

5. Attach black beer disconnects to beer lines & clamp in place.

6. Attach gas bottle bracket to rear of fridge and feed gas line from inside thru hole on upper corner. Grey disconnects will be inside fridge.

7. Attach gas line (heat end in boiling water first) to regulator and clamp.

8. Turn on fridge and set desired temp on front panel.

9. Release head pressure from 1st keg by pulling ring pull on keg lid.

10. Connect beer(black) to “out” on keg. Ensure pressed on fully – should hear click into place.

11. Pour beer until flow slows to a dribble.

12. Ensure gas is off and connect gas to “in” on keg.

13. Turn on gas tap on the bottle, then slowly turn on regulator, rotating in & clockwise.

14. Use minimal gas – just enough to move needle on guage about 1mm only! Don’t let your “beer expert” mates play with it!!!

15. Pour beer & enjoy!

16. Repeat steps 9 to 15 when keg empties. Leave empty kegs pressurized. We will clean them at U-Brew It.

17. If foamy, reduce gas on regulator. If still foamy disconnect gas & beer lines and shake keg with ring pull pulled fully open, allowing excess gas out. Then redo steps 4 to 10.

18. Gas bottle should last at least 12 full brews.